1. SYSTEM ARCHITECTURE & PIPELINED CONTROL DESIGN

This project replaces the traditional multi-cycle control unit of the classic Mano Machine with a high-performance, 3-stage pipelined architecture implemented in Verilog HDL (PÇ1). The architecture decouples the sequential execution flow by partitioning the datapath into parallel stages, allowing multiple instructions to be processed simultaneously in different phases of completion (PÇ1).

The pipelined control unit consists of the following core sub-modules:

Instruction Fetch (IF) Stage: Logic that retrieves the next instruction from the unified Block RAM based on the Program Counter (PC) (PÇ1).

Instruction Decode (ID) Stage: Combinational circuitry that interprets the opcode, resolves memory addresses, and reads the required operands from the register file or memory (PÇ5).

Execute (EX) Stage: The ALU and datapath routing that performs the arithmetic or logical operation and writes the results back to the destination (PÇ1).

Pipeline Registers (IF_ID & ID_EX): Dedicated synchronous registers designed to preserve data, memory addresses, and control signals across clock boundaries, synchronizing the overlapping stages (PÇ1).

Hardware Wrapper (fpga_top.v): A top-level module mapping the processor's clock to the Digilent Basys 3's 100MHz oscillator, utilizing a snooping circuit and a multiplexing switch (SW0) to route final computed variables to the 16 onboard LEDs (PÇ5).

[INSERT IMAGE HERE: The top-level RTL schematic (image_7a41e6.png) showing the CPU wrapped with the RAM and LED multiplexers]

[INSERT IMAGE HERE: The expanded gate-level schematic (image_7a4d62.png) showing the internal complexity and routing of the pipeline]

2. PERFORMANCE ANALYSIS, CRITICAL PATH, AND DESIGN LIMITATIONS

While a pipelined control unit provides superior throughput over multi-cycle designs, it introduces distinct hardware trade-offs, specifically regarding data dependencies and branch latency (PÇ5).

Performance Metrics and Trade-Off Analysis:

Clock Cycles Per Instruction (CPI): A basic memory-reference instruction that takes 4 to 6 clock cycles in a classic multi-cycle unit effectively requires only 1 clock cycle in this pipelined unit after the initial 3-cycle fill stage, assuming no hazards (PÇ5).

Data Hazards (RAW - Read After Write): The pipeline architecture is limited by data dependencies. If an instruction requires a value not yet written back by the preceding Execute stage, the dedicated Hazard Management Unit must deliberately stall the pipeline, forcing the dependent instruction to wait (PÇ1).

Control Hazards (Branching): When branch conditions (BUN, SZA, SNA) are evaluated in the Execute stage, the pipeline has already fetched the next two sequential instructions. The Hazard Management Unit must flush these invalid instructions by injecting NOP (No Operation) commands into the IF_ID and ID_EX registers to prevent data corruption, introducing a minor cycle penalty (PÇ13).

3. TESTBENCH VERIFICATION, SIMULATION & SYNTHESIS RESULTS

Verification of the pipelined processor was conducted through behavioral simulations in Xilinx Vivado using a custom, auto-halting testbench (tb_mano_pipeline.v) (PÇ5). The test program loaded into main memory evaluates a complex 16-bit floating-point addition algorithm written in Mano assembly (fp_add.asm) and converted to machine hex by a custom Python assembler (PÇ1).

Instruction Sequence Executed in RAM:

Operand Loading: The mantissa and exponent components of two floating-point numbers (0x4500 and 0x4100) are loaded into the pipeline (PÇ1).

Masking and Alignment: The ALU executes bitwise masking and logical right shifts to align the smaller mantissa (0280) with the larger exponent (0400) (PÇ1).

Execution & Result: The ALU successfully adds the aligned mantissas, resulting in a Final Exponent (RES_EXP) of 0x4400 and a Final Mantissa (RES_MAN) of 0x0780 (PÇ1).

[INSERT IMAGE HERE: The Tcl Console screenshot showing the formatted math verification RES_EXP : 4400 and RES_MAN : 0780]

Simulation Waveform Execution Trace:

Performance: The 16-bit floating-point addition algorithm completed in exactly 2020 nanoseconds. Based on a 10ns clock period, the pipeline resolved all loops and hazards to execute the full computation in 202 clock cycles, an approximately fourfold speed increase over the multi-cycle equivalent (PÇ5).

Hazard Resolution: Waveform analysis confirms that when the Program Counter (PC) triggers a branch condition, the PC successfully jumps while the intermediate pipeline registers are flushed with NOPs, bypassing invalid executions (PÇ13).

[INSERT IMAGE HERE: The Vivado waveform screenshot showing the pc (Program Counter) jumping and bypassing the flushed instructions during a branch]

Full synthesis and implementation flows completed with zero timing violations, generating the final bitstream file targeting the Digilent Basys 3 FPGA device (PÇ5).

[INSERT IMAGE HERE: The "Bitstream Generation successfully completed" popup screenshot to prove physical synthesizability]

4. TEAMWORK, COLLABORATION AND TASK ALLOCATION